Nonlinear model and its application in FEM analysis of multi-branch cast-in-situ concrete pile

# 1. Introduction:

Numerical modeling is fast, convenient and reliable based on a good model. Finite element is a useful tool in numerical analysis, widely used in solid mechanics and structure analysis.

Finite element method could be used in the soil-structure interaction analysis. The stiffness of soil and structure are determined by their constitutive relationships. Generally, soil has a non-linear strain-stress relationship, in call for some non-linear models in FEM simulation. While the material of structure has a much higher strength compared to soil so it could be taken as linear elastic material.

Considering the relative slip between structure and soil under loading, we should introduce another special “material” constitutive relationship for the interface elements, which could simulate the gap and cracks between structure and soil. The most common model for the interface element is Goodman element.

# 2. Geotechnical models:

The characteristics of soil deformation include non-linearity, elastic-plasticity, dilantacy, hardening and softening, etc.

## 2.1 Geotechnical models:

Some common Geotechnical models:

• Cam Clay

• Drucker-Prager

• Mohr-Coulomb

• Non-linear elastic model

## 2.2 Nonlinear elastic model:

General elastic considerations include continuous, homogeneous, isotropic, linear elastic, etc. Two material properties are needed for stress-strain response: we can use E & µ or G & µ or any other pair of elastic constants. In non-linear analysis, the nonlinear response is divided into several "pieces" or increments, and each step or increment is conducted as linear analysis, so it’s a piecewise linear analysis. Thus, a nonlinear function for each of two elastic constants is required for nonlinear elastic analysis, that is, a non-linear function for Young’s modulus and a nonlinear function for Poisson’s ratio. Various functional forms could be used.

(a)	    	 	    (b)			        (c)		       (d)

(a) Nonlinear with unloading-reloading                              (b) Nonlinear with softening

(c) Linear elastic-perfectly plastic                                       (d) Linear elastic-hardening plastic

Fig.2-1  Typical Nonlinear Material Response

One of the most widely used nonlinear Material Modeling is the hyperbolic Model (Kondner, 1963 (for cohesive soil, subsequently modified), Duncan & Chang 1970 (general model, with strength), Boscardin et al. 1990 (typical parameters)).

Kondner (1963) found the strain-stress relationship under triaxial compression test is close to a parabolic. (Fig.2-1(a)), and εaσ1−σ3~εa is close to a straight line (Fig. 2-1(b)).

εaσ1−σ3=a+bεa     	(2-1)

(a)                                                                     (b)

Fig.2-2  The strain-stress relationship of soil

Where 1/a = initial modulus Ei, 1/b = ultimate strength. Both a & b are material parameters to be determined from tests, and it’s usually expressed in terms of effective stress.

Duncan and Chang (1970) used equation (2-2) to express σ1−σ3~εarelationship:

σ1−σ3=εa1Ei+Rfεaσ1−σ3f                                         	(2-2)

Where Rf is the failure ratio, and generally 0.75 < Rf < 1.0

Rf=σ1−σ3fσ1−σ3ult                                        	(2-3)

Ei is the initial tangent modulus (the initial slope of strain-stress curve), and varies with 3.

Ei=KPaσ3Pan                                                                (2-4)

According to the morh-colomb failure criterion,

σ1−σ3f=2ccosϕ+2σ3sinϕ1−sinϕ                                      	(2-5)

From the above equations, we can get Ei as below:

Et=1−Rf1−sinϕσ−σ32ccosϕ+2σ3sinϕ2KPaσ3Pan                                    	(2-6)

And the modulus of unloading and reloading is as below:

Eur=KurPaσ3Pan      	              (2-7)

Kulhawy et al.(1969) found that the 1 and -3 from conventional triaxial tests also could be described as:

−ε3=μiε11−Dε1                                     	(2-8)

Where I is the initial tangent Poisson ratio, which varies with :

μi=G−Flgσ3Pan                            	 (2-9)

From above equations, we can get:

μt=G−Flogσ3/Pa1−Dσ1−σ3KPaσ3Pan1−Rfσ1−σ31−sinϕ2ccosϕ+2σ3sinϕ2                      (2-10)

Parameters c, , Rf, K, Kur, Kb, m, and n could be achieved from conventional tri-axial tests.

# 3. FEM analysis of multi-branch cast-in-situ concrete pile

## 3.1 Introduction of multi-branch cast-in-situ concrete pile

The cast-in-situ belled concrete piles with multi-branch is a new type of pile foundation.  Compared with the normal uniform diameter bored pile, MULTI-BRANCH pile has higher bearing capacity and better technical and economics effects, thus achieving wide application and getting welcomed in practical projects.

Fig.3-1  The structure of multi-branch concrete pile (in different soil layers)

By the extrusion and expansion, the 3-way arms of the MULTI-BRANCH device in the hole make out the cavities in 3 ways or 3n ways horizontally and symmetrically onto the wall of the hole, then the cavities are filled with concrete to form the branches of MULTI-BRANCH piles for load sharing.  The size and shape of the branches mainly depend on the extruding degree by which the arm of the device creates 3 cavities simultaneously at one time.  To achieve the best bearing capacity, it is important for the branches to be designated with optimized shape and suitable quantity at proper location along shaft.

The bearing capacity, settlement, and economics have been the chief problems in the devise of pile foundations.  This paper will study the factors influencing the bearing capacity of a single MULTI-BRANCH pile as well as the stress and deformation of the ground soil with FEM, to achieve the optimization of MULTI-BRANCH pile design to enhance its bearing capacity, decrease the settlement, and improve the ground stress distribution.

## 3.2 Objectives of FEM analysis

In this paper, the study was made on the mechanism of the single MULTI-BRANCH pile subjected to vertical loading with FEM code THGEA.  Analysis and compare were made on the distribution of deformation and stress in the ground through the vertical stress and vertical displacement contours of different pile types, the load transmission mode of MULTI-BRANCH piles with different branches, and the behavior of branches under work, the bearing behavior of MULTI-BRANCH piles and its influencing factors—branch amount, branch spacing and branch shape, upon which results were drawn for the optimized design and helpful references in practical situations.

## 3.3 Some details in FEM analysis

Code introduction:

THGEA (General Program of Tsinghua Geotechnical Engineering Analysis) is coded specially for the stabilization and deformation analyses of dams and foundations, and could deal with the plane stress, plane strain and axial-symmetry cases.  It has been used to carry out finite element analyses in many practical projects, including the plastically concrete waterproof wall in China Three-Gorge Project, proved to be reliable and satisfactory.

In this paper, a two-dimensional nonlinear elastically FEM analysis was conducted to evaluate the bearing mechanism of MULTI-BRANCH pile and the mechanical behavior of the ground under vertical loads with THGEA.

Basic assumptions:

1.  The ground soil is infinite, uniform, homogenous, and nonlinear elastic material.

2.  The reinforced concrete in pile is continuous and homogenous elastic material.

3.  Under ultimate load, large compression or shear deformations occur in soil, while the concrete pile itself does not fail.

Interface element:

Use Goodman interface element (Goodman et al, 1968) to simulate the relative slip or gap between pile and soil.

Fig.3-2  Goodman interface element

Goodman elements are zero-thick 1-D elements.

There are only shear stresses  (positive for clockwise) and normal stresses  (positive for compression) on the interface, and corresponding relative shear displacement ws and nominal displacement wn. Material parameters include shear stiffness coefficient ks and nominal stiffness coefficient kn.

σ=τσ=ks00knwswn                                 	(2-11)

Let kn be very large (eg. 109kN/m3) when the interface element is under compression to minimize the compressive displacements, and very small under tension (eg. 10kN/m3) to minimize the tensile stresses.  While ks describes the relationship between shear stresses  and shear displacements w, which is as below (Clough & Duncan, 1971):

τ=wa+bw                              	(2-12)

Thus we can get its tangent stiffness coefficient as:

kst=kγwσPan1−Rfτc+σtgϕ	(2-13)

Chosen material parameters for soil and pile:

The ground soil is a uniform medium sand layer, whose Ducan-Chang E-B parameters were achieved from prior tri-axial tests, listed in Table 2-1.  The linear elastic parameters for pile and Goodman interface element parameters are listed in Table 2-2 and Table 2-2 respectively.

| γ (kg/m3) | K (10kPa) | Kb (10kPa) | m |
| --- | --- | --- | --- |
| 1.8 | 423 | 200 | 0.266 |
| n | Kur (10kPa) | c (10kPa) | Rf |
| 0.5 | 500 | 2.5 | 0.7 |

Table 3-1  Duncan-Chang E-B parameters for uniform medium sand as ground soil

| γ (kg/m3) | E (10kPa) | μ |
| --- | --- | --- |
| 2.4 | 2900000 | 0.3 |

Table 3-2  Linear elastic parameters for pile

| Φ (°) | c (10kPa) | Rf | Ks | ns |
| --- | --- | --- | --- | --- |
| 30 | 0.50 | 0.75 | 5000 | 0.70 |

Table 3-3  Goodman interface element parameters

Units: Pa is 10.33, γ is kg/m3, E is 10kPa, load is 10kPa (t).

Pile size:

Diameter d=0.5m, length l=15m, number of branches is 4.

Loadings:

The maximum vertical static load is 4000kN. Add loads from 0 to 4000kN in 14 stages.

| Stage | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage load (kN) | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |
| Total load (kN) | 200 | 400 | 600 | 800 | 1000 | 1400 | 1800 | 2200 | 2600 | 3000 | 3400 | 3600 | 3800 | 4000 |

Table 3-4  Loading stages

Meshes:

Considering the axial-symmetry Geometry and loading mode of the pile foundation, two-dimensional half domain of the whole ground is taken for the FEM meshes, with fixed boundaries on the side surfaces and at the bottom.  Fig.3-3 shows the calculation meshes. (Take the Multi-branch pile with four branches for example.)

Before loading                                                                   After loading

Fig.3-3 Meshes for calculation

## FEM analysis results and discussion

## 3.4.1 Stress and deformation of the ground under vertical loads

# (a) meshes before loading                (b) deformed meshes after loading             (c) soil displacement vectors

# Fig. 3-4  The deformation and displacement of soil and pile elements

Fig.3-4 (b) shows the deformed meshed under vertical loads compared to original meshes before loading in Fig.3-4 (a). Fig.3-4 (c) showed the field of displacement vectors in soil after loading, from which we can know the directions and relative magnitudes of soil displacements.

Fig.3-5 Vertical stress contours comparison        Fig.3-6 Displacement contours comparison

Fig.3-5 and Fig.3-6 show the comparison of contours of vertical stress and displacements of the ground soil between piles with four branches and without branches. Piles compared are in the diameter and of the same length.  The left half of each figure shows the contour of uniform-diameter pile and the right half shows that of multi-branch pile with four branches to make the contrast effects.  The stress contour of Multi-branch pile features the “stress localization zone” ranged from 4d to 6d under the branches and the “stress relief zone” ranged from 2d to 4d above the branches.  Therefore in design the spacing between two branches should be no less than 6d to avoid the overlap of the two zones.

## 3.4.2 Influencing Factors of Bearing Capacity of Multi-branch pile

To study the influence of the branches on the bearing capacity and settlements of Multi-branch piles, FEM calculation was carried out on three influencing factors: branch amount, branch spacing, and branch shape.  By calculating different Multi-branch piles with the same diameter and length but with various branch amounts, spacing and shapes, compare was made according to the load-settlement behavior of them.

To standardize the compares, all piles used in this paper are all with the same diameter of 0.5m, and the same length of 15m if without special denotation, by changing the three influencing factors respectively to make differences.

1. Branch amount

Calculations were carried on Multi-branch piles with 1, 2, 3, 4 branches (D=1.5m, height=1.0m, extruding degree=90°) and two uniform diameter piles (diameter=0.5m and 1.5m).

Fig.3-7.Influence of branch amount on Q~S curves     Fig.3-8. Influence of branch amount on settlement

(B short for branches)                                                 under 4000kN vertical load

From Fig.3-7 and Fig.3-8, the bearing behavior of Multi-branch piles is much better than that of uniform diameter piles, and the bearing capacity of Multi-branch piles is enhanced and the settlement is decreased with the increase of branch number.  But too many branches would not only abate the bearing capacity of each branch due to the overlap of the “stress relief ” and “stress concentration” between two branches, but also cause trouble for field construction and degrade the extruding effects of other branches.

2. Branch spacing

Calculation was carried on Multi-branch piles with two branches (D=1.5m, height=1.0m, extruding degree=90°) with various branch spacing of 1m, 5m, 7m, 10m, 12m and 14m, as well as two uniform diameter piles with diameters of 0.5m and 1.5m.

Fig.3-9 Influence of branch spacing on Q~S curves     Fig.3-10 Influence of branch spacing on settlement

(B short for branches)                                                        under 4000kN vertical load

Fig.3-9 and Fig.3-10 show that the bearing capacity of Multi-branch piles is enhanced and the settlement is decreased with the increasing branch spacing.  Large spacing would cause each branch to work independently, thus abate the disturbance among branches and improve the bearing resistance of each branch.  With the suitable spacing between branches, the deeper the branches are located, the stronger the bearing capacities of them.  The suitable spacing between two branches is no smaller than 3D considering for the good of stress retribution.

3. Branch shape (the extruding degree θ)

Calculations were carried on four-branch Multi-branch piles with differently shaped branches controlled by various extruding degrees of 30º, 60º, 70º, 90º, 150º, and two uniform diameter piles with diameters of 0.5m and 1.5m.

Fig.3-11 Influence of branch shape on Q~S curves            Fig.3-12 Influence of branch shape on settlement

(B short for branches)                                         under 4000kN vertical load

From Fig.3-11 and Fig.3-12, the bearing capacity of Multi-branch piles is improved when the extruding degree decreases, but the improvement slows down after degree is small than 60º.  Counting the limitation of instrument, the suitable degree should be 60º-90º, usually around  70 º, which accords with field tests results well.

## 3.4.3 Load transfer mechanism of Multi-branch piles

In order to design pile foundations with confidence and economy, it is necessary to be familiar with the manner in which the load was transferred from the foundation to the supporting soil under working load conditions.

Fig.3-13 Load transmission of 2-branch pile                            Fig.3-14 Load transmission of 4-branchpile

Fig.3-15 Load transmission of 6-branch pile                        Fig.3-16 Load transmission of 8-branch pile

Following conclusions can be drawn from Fig.3-13—Fig.3-16:

- The sudden changes along the shafts in the places where branches are located show the distinct bearing resistance of branches.

- The less the branches, the greater the sudden changes.  The individual bearing effect of each branch abates when amount increases, and the load transmission mode of 8 branches is near to which of piles with large uniform diameter.

- The shaft friction of Multi-branch piles is lessened above the branches because of the split between the soil and the upper surface of branches.

# 4 Summary

In this paper, study has been made on the mechanism of the single Multi-branch pile under vertical loads with FEM.  Analysis and compare were made on the vertical stress and vertical displacement contours of different pile types, the load transmission mode of Multi-branch piles with different branches, the bearing behavior of Multi-branch piles and its influencing factors—branch amount, branch spacing and branch shape, upon which results were drawn for the optimized design and helpful references in practical situations.

Some worked examples have validated the reliability of the FEM solution to practical problems.  FEM was proved to be economic and flexible, saving much time and work of field tests, and could provide direct and reliable advice for the better design of the pile type in the practical applications, a really helpful tool in foundation studies.

# REFERENCES

- Ergun, M. U. & Akbulut, H (1995), Bearing capacity of shaft-expanded driven model piles in sand, Geotechnique 45, No.4, 715-718, 1995

- Noely, Willian J.  Bearing capacity of expanded-base piles sand. ASCE, Vol.116

- Poulos, H. G. Pile behavior-theory and application. Geotechnique 1989, 39, No.3, pp. 493-502

- Powrie, W.& Li, E.S.F.(1991). Finite element analyses of an in situ wall propped at formation level. Geotechnique 41, No.4, 499-514, 1991

- Xing-long Wu, Guang-mao Li & Zhang-he Wei (2000). Design and analysis for the end-capacity of the single Multi-branch pile. Chinese Journal of Geotechnical Engineering, Vol. 22 No. 5, 581-585

- M.U.Ergun & H.Akbulut(1995). Bearing capacity of shaft-expanded driven model piles in sand. Geotechnique 45, No.4., 715-718

- Bao-han Shen(1979). The Evaluation for Vertical Bearing Capacity of Muti-Reamed Pile. Proc. 3rd. conf. Sol. Mec. Foun. Eng. of CHINA, pp. 354-364